Enterprise Network Design and Implementation Report

Personal Information

- Name: Hazem Hamdi Mahmoud Abdelqader

- University: Nahda University (NUB)

- Faculty: Faculty of Engineering

- Department: Communications and Electronics Engineering

Contact Details

- Phone: +20 106 238 6571

- Email: hazemhamdypersonal@gmail.com

- GitHub: github.com/Hazem-Hamd

1. Introduction

This project focuses on designing and implementing a scalable and secure enterprise network infrastructure using Layer 2 switches, Layer 3 (multilayer) switches, and routers. The network supports multiple VLANs, dynamic IP addressing, redundancy protocols, and secure communication mechanisms.

The design ensures:

- Efficient traffic segmentation using VLANs

- High availability using redundancy protocols

- Secure access control

- Scalable routing using dynamic protocols

2. Network Design Overview

The network consists of:

- Multiple Layer 2 switches for access layer connectivity

- Two Layer 3 switches for distribution/core layer

- Three routers for external and inter-network routing

- A server for testing connectivity and services

- A wireless access point (WAP) for wireless clients

3. Layer 2 Switch Configuration

3.1 VLAN Configuration

VLANs are implemented to segment the network logically:

| VLAN ID | Purpose | Number of Hosts |
| --- | --- | --- |
| VLAN 10 | Department A | 100 Hosts |
| VLAN 20 | Department B | 50 Hosts |
| VLAN 30 | Department C | 25 Hosts |
| VLAN 40 | Department D | 10 Hosts |
| VLAN 99 | Management | Admin Access |

Each VLAN is assigned a separate subnet to improve performance and security.

3.2 Basic Switch Configuration

Each Layer 2 switch is configured with:

- Hostname

- Enable password

- Console password

- VTY (remote access) password

- SSH and Telnet access

Example:

hostname SW1
enable secret cisco

line console 0
password 1234
login

line vty 0 4
password 1234
login
transport input ssh telnet

ip domain-name network.local
crypto key generate rsa

3.3 Port Security

Port security is implemented to prevent unauthorized access:

- Limit number of MAC addresses per port

- Sticky MAC learning

- Violation mode: shutdown

Example:

switchport port-security
switchport port-security maximum 2
switchport port-security mac-address sticky
switchport port-security violation shutdown

3.4 BPDU Guard

BPDU Guard is enabled to protect against rogue switches:

spanning-tree portfast
spanning-tree bpduguard enable

3.5 Wireless Access Point

A wireless access point is connected to the network to provide wireless connectivity. It is configured to:

- Assign clients to a specific VLAN

- Provide secure authentication

- Integrate with DHCP for IP assignment

4. Layer 3 Switch Configuration

Two multilayer switches are used for routing and redundancy.

4.1 Inter-VLAN Routing

SVIs (Switched Virtual Interfaces) are created for each VLAN:

interface vlan 10
ip address 192.168.10.1 255.255.255.0
no shutdown

This allows communication between VLANs.

4.2 HSRP (Hot Standby Router Protocol)

HSRP is used to provide gateway redundancy:

standby 1 ip 192.168.10.254
standby 1 priority 110
standby 1 preempt

- One switch acts as Active

- The other acts as Standby

If the active switch fails, the standby takes over automatically.

4.3 OSPF Routing

OSPF is configured to advertise VLAN networks:

router ospf 1
network 192.168.10.0 0.0.0.255 area 0

This ensures dynamic routing across the network.

4.4 EtherChannel

EtherChannel is used between multilayer switches to increase bandwidth and redundancy:

interface range fa0/1 - 2
channel-group 1 mode active

interface port-channel 1
switchport mode trunk

5. Router Configuration

Three routers are used for network expansion and external connectivity.

5.1 OSPF Configuration

All routers run OSPF for dynamic routing:

router ospf 1
network 10.0.0.0 0.255.255.255 area 0

5.2 Server Connectivity

A server is added to:

- Test network reachability

- Provide services such as HTTP

Ping and HTTP tests are used to verify connectivity.

5.3 Access Control Lists (ACLs)

ACLs are applied to control traffic:

- Allow HTTP traffic

- Allow ICMP (Ping)

- Deny other traffic

Example:

access-list 100 permit tcp any any eq 80
access-list 100 permit icmp any any
access-list 100 deny ip any any

6. DHCP Configuration

Dynamic IP addressing is implemented:

ip dhcp pool VLAN10
network 192.168.10.0 255.255.255.0
default-router 192.168.10.254
dns-server 8.8.8.8

This allows automatic IP assignment to clients.

7. TFTP Configuration

TFTP is used for:

- Backup of device configurations

- IOS image storage

Example:

copy running-config tftp:
copy flash tftp:

This ensures easy recovery in case of failure.

8. Network Security Considerations

The network includes multiple security mechanisms:

- Port Security

- BPDU Guard

- ACL filtering

- SSH for secure remote access

These features protect the network from unauthorized access and attacks.

9. Conclusion

This project demonstrates the design and implementation of a modern enterprise network that is:

- Scalable through VLAN segmentation and OSPF

- Reliable using HSRP and EtherChannel

- Secure with ACLs and port security

- Efficient with DHCP and centralized management

The integration of Layer 2 and Layer 3 technologies ensures high performance and flexibility for future expansion.